4X

NV

adapter

8X

NV-208M

NIKON

DSLR

NIKON Lens

NIKON adapter

1X

CCTV  C-mount

20X Zoom

C-mount

adapter

+25mm F1.17 lens

+216.4mm F2 lens (option)

+100mm F1.5 lens

25mm

100mm

216.4mm

Night Vision Module Adapter

for NIKON DSLR

- The highest performance model for night vision photograph.

- The cutting edge NV-208M incorporates a Gen 2+ tube and combine with catadioptric telescopic lens with super-large aperture openings to convert day-time camera into high resolution image under darkest condition.

- NV-208M comes with 1x and 4x lens and provide 8x lens as option.

- If restrain your camera at ISO 800 and 1/125sec in complete darkness, NV-208M will execute a low light capability to capture a sharp and bright night-time images that can be photographed or video recorded.

- Ideal for wide area surveillance and long distance detection with the configuration of  100mm/216.4mm telephoto catadioptric lens with minimized F-number at 1.5/2 only.

- Permit to use with our IR laser Illuminator to extend the range of 500m while retaining the function of photographing, recording and image stabilization.

Option:

- 216.4mm, F2

lens

- IR illuminator

(500m range)

| Model |  | NV-208M |
| --- | --- | --- |
| NV adapter | Length | 98.5 mm |
|  | Diameter | 66.1 mm |
|  | Weight | Around 342g |
|  | Bayonet mounts | AF-S to Nikon |
|  |  | Mount to objection lens |
|  | Power | From Camera |
| Image intensifier tube | Photocathode operating diameter | 18 mm |
|  | Photocathode type | S-25 |
|  | Resolution | 57-64 Lp/mm |
|  | Sensitivity | 600-750 µA/lm |
|  | Luminous gain | 40,000 fl/fc at 10-6 fcd |
|  | Dark background brightness, max | 1,5x10-3 cd/m2 |
|  | Signal-to-noise ratio | 20-23 |
|  | Current consumption, max | 16mA |
|  | Mean time before failure | 10,000 hours |

Objective Lens

| Magnification | 1x | 4x | 8x |
| --- | --- | --- | --- |
| Optics | Refractor | Catadioptric | Catadioptric |
| Angle | 40° | 10° | 5° |
| Focal length | 25mm (≈75mm/135) | 100mm (≈300mm/135) | 216.4mm (≈600mm/135) |
| Aperture | f 1.17 | f 1.5 | f 2 |
| Outer dia.. | 3.5cm | 8cm | 13cm |

Standard package

RAYSUN NV-208M

(1) NV-208M main unit

(2) 25mm, F1.17 lens

(3) 100mm, F1.5 lens

(4) IR illuminator adapter

(5) Watertight case

Specification：

Performance：